__

\ /          |_

/ \ e n o n  |  o u n d a t i o n

presents:

\     /

*------  the  \ /

/ \  files  ------*

/     \

Spring/1994  Issue: 15

"Stealth in Diverting - PBX Style"

By: Erik Turbo

Disclaimer:

The information provided below is solely for the purpose of diverting

yourself  from possible  traces, ANI, and Caller ID.  It  is *not* to

be used for long distance toll fraud, including abusing 900 services,

using illicit calling cards, or other forms of credit card fraud.

Introduction

~~~~~~~~~~~~

A PBX, or Private Branch eXchange, is an on-premise facility, owned or leased

by an organization,  which interconnects  the telephones  within the facility

and  provides  access  to the  public  telephone  system.  Basically, it is a

mini-switching station,  and  allows a telephone user on the premises to dial

a  three  to four digit  number (extension) to  call another telephone on the

premises,  and dials  one digit  (usually 8 or 9)  to get a  dial tone for an

"outside line,"  which  allows  the  caller to  dial  out to  the rest of the

public  telephone user.  This is the most important feature for a hacker that

desires the stealth that is necessary to continue his explorations. In short,

you can remain well-hidded if you use a PBX's outside lines to connect to the

computer you are hacking.  That way,  any Caller ID, ANI, or trace will reach

the PBX number - not your home telephone.  When  you have mastered the art of

PBX hacking,  you should  make a habit of diverting with 3 or 4 "well spaced"

PBX's  before  hitting your  target  destination.  For  the  advanced hacker,

diverting  with PBX's  is just  the beginning  of his actual diversion; it is

best  to bury  yourself  in  packet-switched  networks,  loop  in  and out of

Internet hosts, bounce yourself off of satellites with International calling,

and  utilize all of the  data-based outdials that you have. Remember, abusing

these PBX's for un-necessary long distance calling is NOT condoned by me,  or

any members of the Xenon Foundation; it will kill the PBX quicker,  and place

you at risk of serious fraudulant charges.

Definity G Model System 75

~~~~~~~~~~~~~~~~~~~~~~~~~~

Definity  model  System 75 systems  control a large number of medium-sized

(approx. 1000 lines)  PBX's.  It  is owned  by AT&T,  and was developed in

the late 1970's, with modifications in 1983,  and 1986.  The actual System

75  machine has  one or  more incoming  1200bps data lines,  which connect

at 7E1.  It  is  through  this  remote port that you may begin your actual

hacking of the PBX.  Since all of the  changes you  may via  modem  affect

the  entire  telephone  network on the  PBX, this is a power that you will

have  to learn  how to  abstain from  abusing.  It is possible to turn the

once  smoothly operating phone system into a chaotic mass of busy signals,

re-routes, Voice Mail Box's, tones, and bridges, effectively shutting down

the victim for hours,  if not days.  For this  reason,  I will only inform

readers on how to create a remote extension for diverting purposes.

Connection

~~~~~~~~~~

The  best way  to find a System 75 is to scan ("wargame dial") your local

telephone exchanges.  There  are  still  dozens of  them around,  and you

are bound to hit at least one in a few days of scanning.  Upon connection

you will see the System 75 login and authorization prompts:

Login: xxx

Password: xxx

INCORRECT LOGIN

You  will be given  three chances  to guess authorization password before

the  system  will  drop  carrier.  On  telephone   systems  that  provide

Caller ID services,  I would be weary; it is quite possible the System 75

dialup as well as the PBX are equipt with ANI for auditing purposes.

Default Accounts and Passwords

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

With  every  new  System  75 package,  AT&T  includes a  large  number of

accounts  and passwords  already installed and ready for usage.  Usually,

these passwords are never changed by the owners. Here is a listing of the

known System 75 default accounts and passwords that are included in every

Definity G package:

Login: enquiry  Password: enquirypw

Login: init     Password: initpw

Login: browse   Password: looker

Login: maint    Password: rwmaint

Login: locate   Password: locatepw

Login: rcust    Password: rcustpw

Login: tech     Password: field

Login: cust     Password: custpw

Login: inads    Password: inads

Login: support  Password: supportpw

Login: bcim     Password: bcimpw

Login: bcnas    Password: bcnspw

Login: craft    Password: craftpw

Note: The browse account can *not* modify anything on the system 75.

It is only useful for examining the possibility of an existing

remote extension, not for the actual creation of one.

Internal System 75 Commands

~~~~~~~~~~~~~~~~~~~~~~~~~~~

If those  defaults did not work,  the only other alternatives is social

engineering,  and brute  force  hacking.  Both  are not  likely to work

unless you are a social engineering master, or have time to waste brute

force  hacking.  Your  best bet is to move on and hope to scan a lesser

protected System 75.

Assuming  you have passed  the authorization, you will be prompted with

the following:

Terminal Type (513, 4410, 4425): [513]

These  are the  basic  System 75  remote  usage emulation codes.  I prefer

to use 4410,  as  it  appears  much  "cleaner"  on  a  VT100  IBM PC.  The

emulation  is  used  to remotely send System 75 key sequences,  to request

help, to save a session, to move forward a page, to move back a page, etc.

Since  the  IBM keyboard  has no way  to emulate these keys, the System 75

provides  3  basic  emulation  codes.  For  our  purposes,  use 4410.  The

following sequences will work with emulation 4410:

ESC Op - To cancel a command

ESC Ot - To request Help

ESC Ov - Next Page

ESC Ow - Previous Page

ESC Or - Save

ESC Oq - Refresh Screen

ESC Os - Clear Fields

You can achieve  the escape sequences by hitting the ESC key, and then the

key combination O and the following character.

Once you have choses emulation 4410, please remember (or take note) of the

previous  escape  sequences.  You  will  not  be able  to save information

without knowing the proper code. ("ESC Or"). ESC Op is also very important

since  it  is  the  only  method  of  stopping the execution of a command;

something  you will have to do constantly when looking over certain pieces

of information within the System 75.  Take these down!

Next you will see the AT&T banner and the command prompt:

Copyright (c) 1986 - AT&T

Unpublished & Not for Publication

All Rights Reserved

enter command:

There is online help avaiable at all times by pressing "ESC Ot", as well

as  keying 'help' at  the command prompt.  Familiarize yourself with the

system.  It  is  basically  cryptic,  as  it  is  usually  only  used by

experienced AT&T technicians.

Examining the PBX

~~~~~~~~~~~~~~~~~

Once you are in, you now want to get to working on your  diverter.  What

you will obviously need is an extension dedicated  explicitly for a dial

tone  to the  outside  network.  To accomplish  this quickly and easily,

all you must do is type  "change remote" at  the  command  prompt.  This

will bring you to the following screen:

change remote-access                                           Page  1 of  1

REMOTE ACCESS

Remote Access Extension:

Barrier Code Length:

BARRIER CODE ASSIGNMENTS (Enter up to 10)

Barrier Code    COR                    Barrier Code    COR

1:                 1                    6:                 1

2:                 1                    7:                 1

3:                 1                    8:                 1

4:                 1                    9:                 1

5:                 1                    10:                1

As you can see, there is no remote access extension set up, therefor this

PBX  does not  have any existing dialtones available.  Now to create one,

type  in the  extension  you  wish  to  direct you  to your dialtone. The

extension you type in should be a 4 digit number, startin with "2" or "4"

as  these are valid  extensions  under System 75 software.  When you type

in your extension, press enter; if it gives you an error, try a different

extension until it accepts your input.

If  you wish  to add a security code on your dialtone, you may enter it's

length at  the "Barrier Code Length:" prompt.  Under the heading "Barrier

Code", at the "1:" prompt, type in your desired security code.  After you

are all set, the screen should look something like this:

change remote-access                                           Page  1 of  1

REMOTE ACCESS

Remote Access Extension: 2400

Barrier Code Length: 6

BARRIER CODE ASSIGNMENTS (Enter up to 10)

Barrier Code    COR                    Barrier Code    COR

1: 222222          1                    6:                 1

2:                 1                    7:                 1

3:                 1                    8:                 1

4:                 1                    9:                 1

5:                 1                    10:                1

Now you have a working extension that is not only available for your

use in diverting,  but also secure  from others who do not know your

barrier code.  Type the key combination "ESC Or" to save your work.

Finding the PBX Dialup

~~~~~~~~~~~~~~~~~~~~~~

Now that you are guarenteed a tone, you must find out the telephone number

the  PBX is  located at.  Type  "list trunk-group"  at the command prompt.

It should give you a listing similar to this:

Group                                       No. of                  Outgoing

Number  TAC  Group Type    Group Name       Members   COR   SMDR?   Display?

1       801   co           Incoming          12        1     y        n

2       851   co           Sales Room         1        1     y        n

9       809   co           Billing            4        1     y        n

10      810   co           Admin line         1        63    y        n

Command successfully completed CANCEL         P                                                           HELP          T

Now that you have a listing of all the trunk groups that are present on the

PBX,  you can  individually  list them to get their corresponding telephone

numbers.   Type   "display  trunk-group 1",   to  display   trunk  group  1

(Group Number 1,  Group Name  "Incoming").  As  you  can see from the above

capture,  there  are  4 trunks available; 1, 2, 9, and 10.  Display each of

them,  and use  the  'next page' ("ESC Ov")  key  combination to get to the

page  (usually page 2, or 3) with  the telephone numbers to the trunk. Each

time  you  display  the  trunks,  you  will  get  a  screen  similar to the

following:

display trunk-group 1                                           Page  1 of  5

TRUNK GROUP

Group Number: 1                    Group Type: co            SMDR Reports? y

Group Name: Incoming                    COR: 1                      TAC: 801

Direction: two-way        Outgoing Display? n         Data Restriction? n

Dial Access? y                Busy Threshold: 60           Night Service:

Queue Length: 0                                      Incoming Destination: 200

Comm Type: voice                                 Digit Absorption List:

Prefix-1? n                   Restriction: toll    Allowed Calls List? n

TRUNK PARAMETERS

Trunk Type: loop-start

Outgoing Dial Type: tone

Trunk Termination: rc                    Disconnect Timing(msec): 500

ACA Assignment? n

Maintenance Tests? y

Answer Supervision Timeout:                    Suppress # Outpulsing? n

_____________________________________________________________________________

To get the actual dialups, you must look on the following pages. The

"ESC Ov" combination will do that under emulation 4410:

_____________________________________________________________________________

display trunk-group 1                                           Page  2 of  5

TRUNK GROUP

GROUP MEMBER ASSIGNMENTS

Port      Name         Mode         Type    Answer Delay

1: A0101    555-2322

2: A0102    555-2342

3: A0103    555-2343

4: A0104    555-2345

5: A0105    555-2456

6: A0106    555-2457

7: A0107    555-2458

8: A0108    555-2459

9: A0201    555-2460

10: A0202    555-2461

11: A0203    555-2462

12: A0204    555-2470

13: A0205    555-2800

14: A0206    555-2810

15: A0207    555-2811

Make a note  of the telephone numbers on the trunks, and dial  them up after

logging  off  the  System  75.  When  you  dial  them  up  voice,  if one of

them prompts you for an extension, type in the remote extension  you created

earlier.  You should hear the tone to an outside line.  If  you created  the

remote extension with a barrier code, touch-tone that in now. Next, dial "9"

to  get an outside line (It can also be "8" on some systems),  and then dial

the  telephone number  you want to reach, just as normally as you would from

your home telephone.

Tricks and Hints

~~~~~~~~~~~~~~~~

The following are methods and commands that can be used in addition to

the  above mentioned  hacking tactics.  They  are not necessary to the

smooth creation of a remote dialtone off of a PBX, however.

When  you are displaying the trunk-groups individually, look under the

heading  "Direction: "  (found on page 1).  If it says "one-way", then

modify that (with the "change trunk #" command) to say "two-way". Also

on page 1, change the "Incoming Destination: "  header to reflect your

newly  created  remote  access extension  that you created earlier. On

the  next page,  get  the  dialups.  You have just created a large set

of  tones.  Since  they  used  to be  "one-way", only users within the

building  could use  them to dialout, but since you have changed it to

"two-way",  and  changed the  incoming  destination  extension to your

remote extension,  you are  allowing incoming  callers to use the tone

service as well.

If you do  not want to arouse suspicion, instead of changing the "Incoming

Destination: " to your extension, just change the "Night Service: " header

to your remote  extension.  With this,  however, you can only use the tone

service after hours; usually after the business closes.

To  get  an  idea  of how the extensions are uniformly placed on the PBX,

type  "display dialplan" at  the  command  prompt.   This  will  give you

all the prefix's to the three or four digit extensions.  This is valuable

if  you are  having  trouble  finding  a valid  extension to use for your

remote extension.

When  displaying  a trunk  group, mark down the COR (Class of Restriction)

number.  Type  "display COR #"  (where #  is the  COR number of a specific

trunk). Make sure the FRL prompt is set to 7, and the calling restrictions

are  set to  "none".  If not,  type  "change cor  (COR #)",  and  make the

necessary modifications.

Type "display feature" to get a listing of all the feature access codes on

the system. This is valuable if you can not get an outside line by dialing

"9", or "8".  The dialout access code will be in here.

Conclusion

~~~~~~~~~~~

Basically,  it is  extreemly simple  to create a remote extension off of a

PBX  in  your local area.  If you use the PBX just to make local calls and

to divert  yourself further  through the telephone network, it should last

a  rather  long  time.   However,  if  you  abuse  it by dialing  Alliance

Teleconferences every night, or to call your friend in the UK  three times

a day, it will either die, or get slapped with ANI.

I  have  tried to be  as straight  forward  as possible, without having to

technically  explain  every detail of operation.  Once you get the hang of

it,  you should  be able to  create  your  remote  extensions  in under 10

minutes.  However,  if you  are having problems, you can contact me at the

following locations:

Internet Mail: erikt@xf.com

Void of Deception: [508]/998-2400

Additional Reading:

~~~~~~~~~~~~~~~~~~~

Hacking AT&T System 75, Scott Simpson, Phrack 41, File 6.

System 75 Hacking (An Online Tutorial), Panther Modern, COTNO01.TXT, File 3.

Data and Computer Communications, William Stallings, Macmillan Publishing Co.